CARO_Platform

Telemetry Path Reference

From MQTT Broker to Screen Pixel

April 2026  |  Version 1.0

Internal Technical Reference — Not for Distribution

# 1. Overview

This document traces every step a telemetry value takes from the MQTT broker to a rendered pixel on the operator’s screen. Each section corresponds to one module in the codebase. Code snippets are taken verbatim from the implementation; line references are included. The goal is to leave zero ambiguity about how data flows, transforms, and is consumed.

## 1.1 End-to-End Summary

The full path has seven stages:

| # | Stage | Module | What Happens |
| --- | --- | --- | --- |
| 1 | MQTT Publish | mqtt-simulator | Device publishes JSON telemetry to caro/{module_id}/telemetry at ~10 Hz |
| 2 | MQTT Receive | mqtt-bridge.ts | HMI server subscribes to caro/+/telemetry, parses JSON, iterates tags |
| 3 | LKV Cache Write | lkv.ts | Value written to in-memory map. Generation bumped only if value differs (strict ===) |
| 4 | DB Pipeline Enqueue | db-pipeline.ts | Changed trendable tags enqueued with module-level timestamp for TimescaleDB |
| 5 | WebSocket Tick | ws-server.ts | Every 125 ms, per-client generation diff; DELTA JSON sent for changed subscribed tags |
| 6 | Client Context | HmiContextProvider.tsx | Browser WebSocket receives DELTA, updates valuesRef, calls subscriber callbacks |
| 7 | Widget Render | NumericMon.tsx (example) | useLiveValue re-renders widget with new value; null = bad quality renders "---" |

## 1.2 Data Shape at Each Boundary

The telemetry value changes shape as it crosses each boundary. This table shows the exact type at each stage:

| Boundary | Shape | Notes |
| --- | --- | --- |
| MQTT wire | JSON: { tag_id: number, value: number ¦ boolean ¦ string } | Part of tags[] array inside telemetry message |
| LKV entry | { value: number ¦ boolean ¦ string ¦ null, generation: number } | null = bad quality. Generation is monotonic uint32 |
| DB pipeline entry | { moduleTs: number, tags: [{ tagId, value }] } | module-level timestamp from MQTT message, not per-tag |
| WebSocket wire | JSON: { type: "DELTA", values: { [tag_id]: value } } | Value only. No timestamp, no quality enum |
| React state | LiveValue: { value: number ¦ boolean ¦ string ¦ null } | null = bad quality. Drives widget re-render |

# 2. Stage 1: MQTT Publish (Device / Simulator)

The MQTT Simulator (or a real embedded device) publishes a JSON message to the Mosquitto broker on topic caro/{module_id}/telemetry at approximately 10 Hz (every ~100 ms). QoS 0, no retain.

## 2.1 Message Format

The exact JSON structure published to the wire:

// Published to: caro/{module_id}/telemetry

// QoS: 0, Retain: false

{

"timestamp": 1712617200000,    // Date.now() at publish time (ms)

"status": "ONLINE",            // "ONLINE" or "FAULT"

"tags": [

{ "tag_id": 1, "value": 85.47 },

{ "tag_id": 2, "value": 12.03 },

{ "tag_id": 3, "value": true },

{ "tag_id": 4, "value": 150.0 }

]

}

## 2.2 Key Points

timestamp is the only timestamp in the entire pipeline. It is a module-level value — one timestamp shared by all tags in this message. It is NOT stored per-tag. It is used only by the DB pipeline for TimescaleDB writes.

status can be "ONLINE" or "FAULT". If "FAULT", the HMI server writes null to the LKV for every tag in that module, signaling bad quality.

tags[] contains every tag the module publishes on every tick. All tags (monitor and setpoint) are included. Each tag carries only tag_id and value — no metadata, no timestamp.

value types: number (f64, i32), boolean (bool), or string (str). Matches the data_type from the tag registry.

# 3. Stage 2: MQTT Bridge (mqtt-bridge.ts)

The MQTT bridge is the server-side entry point for telemetry. It connects to Mosquitto, subscribes to caro/+/telemetry, and routes incoming messages to the LKV cache and DB pipeline.

## 3.1 Subscription

On MQTT connect, the bridge subscribes to two topic patterns:

// mqtt-bridge.ts — MqttBridge.start()

this.client!.subscribe('caro/+/telemetry', () => {});

this.client!.subscribe('caro/+/cmd_ack', () => {});

The + wildcard matches any module_id. This means the bridge receives telemetry from all modules on a single subscription.

## 3.2 Message Parsing

Every MQTT message arrives as a Buffer. The bridge extracts the module_id from the topic, parses the JSON, and validates the structure:

// mqtt-bridge.ts — MqttBridge.handleMessage()

private handleMessage(topic: string, payload: Buffer): void {

const parts = topic.split('/');

const moduleId = parts[1];     // "caro/{moduleId}/telemetry"

let message: TelemetryMessage;

try {

message = JSON.parse(payload.toString()) as TelemetryMessage;

} catch {

return;                       // Silently drop malformed JSON

}

this.lastSeen.set(moduleId, Date.now());

this.timedOutModules.delete(moduleId);

Note: lastSeen is updated on every message. This is how the watchdog tracks module health.

## 3.3 FAULT Handling

If the message status is "FAULT", every tag belonging to that module is set to null in the LKV. No values are written. No DB entries are created.

if (message.status === 'FAULT') {

for (const tagId of this.moduleTagIds.get(moduleId) ?? []) {

this.lkv.set(tagId, null);  // null = bad quality

}

return;

}

## 3.4 Tag Iteration and LKV Write

For each tag in the message, the bridge writes to the LKV cache. The lkv.set() call returns a boolean indicating whether the value actually changed. Only changed trendable tags are enqueued for the DB pipeline.

const changedTrendable: { tagId: number;

value: number | boolean | string | null }[] = [];

for (const { tag_id, value } of message.tags) {

if (!this.tagMap.has(tag_id)) continue;  // skip unknown tags

const changed = this.lkv.set(tag_id, value);

if (changed && this.trendableTagIds.has(tag_id)) {

changedTrendable.push({ tagId: tag_id, value });

}

}

if (changedTrendable.length > 0) {

this.dbPipeline.enqueue({

moduleTs: message.timestamp,

tags: changedTrendable

});

}

Note: The tagMap.has() check silently drops tags that are not in the registry. This can happen if a device publishes tags from an old firmware version.

## 3.5 Watchdog

A periodic timer checks if any module has gone silent. If no telemetry has been received for watchdogTimeoutMs (default: 5000 ms), the watchdog writes null to the LKV for every tag in that module:

// mqtt-bridge.ts — MqttBridge.watchdogTick()

watchdogTick(): void {

const now = Date.now();

for (const [moduleId, tagIds] of this.moduleTagIds) {

const last = this.lastSeen.get(moduleId) ?? 0;

const timedOut = now - last > this.config.watchdogTimeoutMs;

if (timedOut && !this.timedOutModules.has(moduleId)) {

this.timedOutModules.add(moduleId);

for (const tagId of tagIds) {

this.lkv.set(tagId, null);  // null = bad quality

}

}

}

}

Note: timedOutModules prevents the watchdog from spamming null writes on every tick after a module disconnects. It re-arms when the module resumes telemetry (lastSeen is updated, timedOutModules.delete is called).

# 4. Stage 3: LKV Cache (lkv.ts)

The Last-Known-Value cache is the central data structure. It is an in-memory Map<tag_id, LkvEntry> holding the most recent value for every tag.

## 4.1 Data Structure

// lkv.ts

export interface LkvEntry {

value: number | boolean | string | null;

generation: number;

}

export class LkvCache {

private entries = new Map<number, LkvEntry>();

There are two fields per entry:

value: The raw telemetry value. null means bad quality (device offline, not yet heard from, or FAULT status).

generation: A monotonic counter that bumps only when value changes. This is the mechanism the WebSocket server uses to detect which tags have new data for each client.

## 4.2 The set() Method — Generation Semantics

This is the most important method in the entire telemetry path. Understanding it is critical to understanding how change detection works.

// lkv.ts — LkvCache.set()

set(tagId: number, value: number | boolean | string | null): boolean {

const existing = this.entries.get(tagId);

if (existing !== undefined && existing.value === value) {

return false;           // Same value — no generation bump

}

if (existing !== undefined) {

existing.value = value;

existing.generation++;  // Value changed — bump generation

return true;

}

this.entries.set(tagId, { value, generation: 1 }); // First write

return true;

}

### Critical Behavior

Strict equality (===): If the device sends 85.47 twice in a row, the second write returns false and generation does not change. This means the WebSocket server will not send a DELTA for this tag — no unnecessary traffic.

null transitions: If value was 85.47 and the watchdog writes null (device disconnected), generation bumps. The WebSocket server will send a DELTA with value: null, and the widget will render "---". When the device reconnects and sends 85.47 again, generation bumps again, and the widget restores the value.

No timestamp: The LKV does not store timestamps. The only temporal concept is generation order, which is monotonic and never wraps in practice (uint32 ceiling is ~4 billion, at 10 Hz that is 13+ years).

Return value: true means the value changed and generation was bumped. false means it was a no-op. The MQTT bridge uses this return value to decide whether to enqueue the tag in the DB pipeline.

# 5. Stage 4: DB Pipeline (db-pipeline.ts)

The DB pipeline is a push-based queue that receives changed trendable tags from the MQTT bridge and batches them for TimescaleDB writes. In the current implementation this is a placeholder — the queue is flushed silently without writing to a database.

## 5.1 Entry Shape

// db-pipeline.ts

export interface DbWriteEntry {

moduleTs: number;    // Module-level timestamp from MQTT message

tags: {

tagId: number;

value: number | boolean | string | null;

}[];

}

moduleTs is the timestamp from the MQTT telemetry message. It is NOT the time the value was received by the HMI server. It is NOT per-tag. Every tag in a single telemetry message shares the same moduleTs.

## 5.2 Enqueue and Flush

// db-pipeline.ts — DbPipeline

enqueue(entry: DbWriteEntry): void {

if (this.queue.length >= this.maxQueueSize) {

this.queue.shift();   // Drop oldest on overflow

console.warn('[DbPipeline] Queue full, dropping oldest entry');

}

this.queue.push(entry);

}

flush(): DbWriteEntry[] {

const batch = this.queue.splice(0);

// Placeholder — silent flush.

// Real implementation writes to TimescaleDB.

return batch;

}

The flush timer runs in index.ts on a configurable interval (default: 1000 ms):

// index.ts

const dbFlushTimer = setInterval(

() => dbPipeline.flush(),

config.dbTickMs    // default: 1000

);

Note: The DB pipeline is completely independent from the WebSocket pipeline. They share the LKV as the single source of truth but run at different rates and are driven by different triggers (push from MQTT vs. pull on timer tick).

# 6. Stage 5: WebSocket Server (ws-server.ts)

The WebSocket server is the bridge between the server-side LKV cache and browser clients. It operates on a pull-based tick pattern: every 125 ms (8 Hz), it checks which tags have changed for each client and sends DELTA messages.

## 6.1 Client State

Each connected WebSocket client is tracked with this state object:

// ws-server.ts

interface WsClient {

ws: WebSocket;

subscriptions: Set<number>;       // tag_ids this client cares about

lastSentGen: Map<number, number>; // last generation sent per tag_id

}

subscriptions is the set of tag_ids the client has asked for via SUBSCRIBE messages.

lastSentGen maps each subscribed tag_id to the LKV generation that was last sent to this client. On each tick, this is compared against the LKV’s current generation to determine what’s new.

## 6.2 SUBSCRIBE → SNAPSHOT

When a client sends a SUBSCRIBE message, the server immediately responds with a SNAPSHOT of the current LKV values for the newly subscribed tags. This ensures the client has initial state without waiting for a delta tick.

// ws-server.ts — WsServer.handleMessage()

case 'SUBSCRIBE': {

const newTagIds = msg.tagIds.filter(

id => !client.subscriptions.has(id)

);

for (const id of msg.tagIds)

client.subscriptions.add(id);

// Snapshot only the newly subscribed tags

const values: Record<string,

number | boolean | string | null> = {};

for (const id of newTagIds) {

values[String(id)] = this.lkv.getValue(id);

client.lastSentGen.set(id,

this.lkv.getGeneration(id)

);

}

this.send(client, { type: 'SNAPSHOT', values });

break;

}

Note: After sending the SNAPSHOT, lastSentGen is set to the current generation. This means the next tick() will only send changes that occurred AFTER the snapshot.

## 6.3 The Tick Loop — DELTA Generation

This is the core of the real-time pipeline. Every 125 ms, for each connected client:

// ws-server.ts — WsServer.tick()

private tick(): void {

for (const client of this.clients) {

if (client.ws.readyState !== WebSocket.OPEN) continue;

const delta: Record<string,

number | boolean | string | null> = {};

for (const tagId of client.subscriptions) {

const currentGen =

this.lkv.getGeneration(tagId);

const lastGen =

client.lastSentGen.get(tagId) ?? 0;

if (currentGen > lastGen) {

delta[String(tagId)] =

this.lkv.getValue(tagId);

client.lastSentGen.set(tagId, currentGen);

}

}

if (Object.keys(delta).length > 0) {

this.send(client,

{ type: 'DELTA', values: delta }

);

}

}

}

### Wire Format

Both SNAPSHOT and DELTA use the same JSON shape:

// SNAPSHOT (sent on subscribe)

{ "type": "SNAPSHOT", "values": {

"1": 85.47, "2": 12.03, "3": true, "4": null

}

}

// DELTA (sent on tick when values change)

{ "type": "DELTA", "values": {

"1": 86.12

}

}

Keys are string tag_ids. Values are the raw value (number, boolean, string, or null). There is no timestamp, no quality enum, no generation counter on the wire. The client receives the value and renders it.

## 6.4 Why Generation Counters?

Alternative approaches and why they were rejected:

| Approach | Problem | Generation Advantage |
| --- | --- | --- |
| Dirty-flag set | One flag per tag, not per client. If Client A reads before Client B, B misses the change. | Per-client tracking via lastSentGen. No client can miss an update. |
| Timestamp comparison | Requires synchronized clocks. Timestamp must be stored per tag. | No clock dependency. Generation is monotonic and local. |
| Broadcast all | Every client gets every tag on every tick. Wastes bandwidth. | Tailored per-client deltas. Only changed + subscribed tags sent. |

# 7. Stage 6: Client Context (HmiContextProvider.tsx)

The HmiContextProvider is a React context provider that wraps the entire HMI application. It manages the WebSocket connection, tag map, and live value subscriptions. It is the bridge between the server’s WebSocket and the React component tree.

## 7.1 Initialization Sequence

On mount, the provider:

1. Fetches the tag map from GET /api/v1/tags. This is a one-time REST call that loads every TagDef into a Map<tag_id, TagDef>.

2. Once the tag map is loaded, opens a WebSocket connection to the server.

3. Renders null until the tag map is loaded. This prevents widgets from mounting and calling useResolveAssetPath on an empty map.

// HmiContextProvider.tsx

if (!tagMapLoaded) return null;

return (

<HmiContext.Provider value={contextValue}>

{children}

</HmiContext.Provider>

);

## 7.2 WebSocket Message Handling

When a SNAPSHOT or DELTA arrives, the provider updates its in-memory valuesRef and notifies all subscriber callbacks:

// HmiContextProvider.tsx — ws.onmessage

if (msg.type === 'SNAPSHOT' || msg.type === 'DELTA') {

const entries = msg.values ?? {};

for (const [key, rawValue] of

Object.entries(entries)) {

const tagId = Number(key);

const lv: LiveValue = {

value: rawValue as

number | boolean | string | null

};

valuesRef.current.set(tagId, lv);

subscribersRef.current.get(tagId)

?.forEach(cb => cb(lv));

}

}

valuesRef is the client-side LKV. It stores the latest LiveValue for each tag_id.

subscribersRef maps tag_id to a Set of callback functions. Each callback is a React useState setter from a useLiveValue hook. When the callback fires, the widget re-renders.

## 7.3 Subscription Batching

When multiple widgets mount in the same render cycle, each calls subscribeLiveValue. Rather than sending a separate SUBSCRIBE message per widget, the provider batches them using queueMicrotask:

// HmiContextProvider.tsx — subscribeLiveValue()

if (!subscribedOnServerRef.current.has(tagId)) {

subscribedOnServerRef.current.add(tagId);

pendingSubscribeRef.current.add(tagId);

if (!flushScheduledRef.current) {

flushScheduledRef.current = true;

queueMicrotask(() => {

flushScheduledRef.current = false;

const batch = Array.from(

pendingSubscribeRef.current

);

pendingSubscribeRef.current.clear();

const ws = wsRef.current;

if (batch.length > 0 && ws !== null

&& ws.readyState === WebSocket.OPEN) {

ws.send(JSON.stringify({

type: 'SUBSCRIBE',

tagIds: batch

}));

}

});

}

}

Note: queueMicrotask fires after all synchronous hooks have mounted in the current React render pass, but before the browser paints. This means 15 widgets mounting simultaneously produce ONE SUBSCRIBE message with 15 tag_ids, not 15 individual messages.

## 7.4 Reconnection

If the WebSocket disconnects, the provider reconnects with exponential backoff (starting at 1s, capping at 30s). On reconnect, it re-subscribes all currently-subscribed tag_ids in a single SUBSCRIBE message.

## 7.5 Unsubscription

When a widget unmounts, its useLiveValue cleanup function fires, which calls the unsubscribe function returned by subscribeLiveValue. When the last subscriber for a tag_id is removed, an UNSUBSCRIBE message is sent to the server:

// HmiContextProvider.tsx — unsubscribe cleanup

return () => {

const subs = subscribersRef.current.get(tagId);

subs?.delete(callback);

if (subs !== undefined && subs.size === 0) {

subscribersRef.current.delete(tagId);

subscribedOnServerRef.current.delete(tagId);

const ws = wsRef.current;

if (ws !== null

&& ws.readyState === WebSocket.OPEN) {

ws.send(JSON.stringify({

type: 'UNSUBSCRIBE',

tagIds: [tagId]

}));

}

}

};

# 8. Stage 7: Widget Render

Widgets are the final destination for telemetry data. They convert a LiveValue into a visual representation on screen. This section traces the path through a NumericMon widget as a concrete example.

## 8.1 Asset Path Resolution

Widgets receive an assetPath string, not a tag_id. The resolution chain is:

Step 1: useSingleTag(assetPath, widgetName) — calls useResolveAssetPath and expects exactly one match.

// shared/useSingleTag.ts

export function useSingleTag(

assetPath: string, widgetName: string

): TagDef {

const tags = useResolveAssetPath(assetPath);

if (tags.length === 0) {

throw new Error(

`${widgetName}: no tags found matching

assetPath "${assetPath}"`

);

}

if (tags.length > 1) {

throw new Error(

`${widgetName}: ambiguous assetPath

"${assetPath}" matched ${tags.length}

tags.`

);

}

return tags[0];

}

Step 2: useResolveAssetPath(assetPath) — does contiguous segment matching against tag_path values in the tag map.

// hooks/useResolveAssetPath.ts

export function useResolveAssetPath(

assetPath: string

): TagDef[] {

const tagMap = useTagMap();

return useMemo(() => {

const assetSegments = assetPath.split('.');

const results: TagDef[] = [];

for (const tag of tagMap.values()) {

const tagSegments = tag.tag_path.split('.');

if (assetSegments.length > tagSegments.length)

continue;

let found = false;

for (let i = 0;

i <= tagSegments.length

- assetSegments.length; i++) {

let match = true;

for (let j = 0;

j < assetSegments.length; j++) {

if (tagSegments[i + j]

!== assetSegments[j]) {

match = false;

break;

}

}

if (match) { found = true; break; }

}

if (found) results.push(tag);

}

return results;

}, [assetPath, tagMap]);

}

Example: assetPath "RF_Fwd.monitor" matches tag_path "Plant1.Module.RF_Fwd.monitor" because "RF_Fwd" and "monitor" appear as contiguous segments.

## 8.2 Live Value Subscription

Once the tag is resolved, the widget subscribes to its live value:

// hooks/useLiveValue.ts

export function useLiveValue(

tagId: number

): LiveValue {

const ctx = useHmiContext();

const [liveValue, setLiveValue] =

useState<LiveValue>(

() => ctx.getLiveValue(tagId)

);

useEffect(() => {

setLiveValue(ctx.getLiveValue(tagId));

const unsubscribe =

ctx.subscribeLiveValue(tagId, setLiveValue);

return unsubscribe;

}, [tagId, ctx]);

return liveValue;

}

The callback passed to subscribeLiveValue is setLiveValue itself — the React state setter. When the provider receives a DELTA from the WebSocket, it calls this setter directly, which triggers a re-render of the widget.

## 8.3 Rendering (NumericMon)

The complete NumericMon widget:

// NumericMon.tsx

export function NumericMon(

{ assetPath, label }: NumericMonProps

) {

const tag = useSingleTag(assetPath,

'NumericMon');

const lv = useLiveValue(tag.tag_id);

const decimals = resolveDecimalPlaces(tag);

const displayLabel =

resolveLabel(assetPath, label);

const badQuality = lv.value === null;

return (

<div className="inline-flex flex-col p-2

rounded border border-gray-200 bg-white

min-w-[80px]">

<WidgetLabel label={displayLabel}

unit={tag.unit} />

{badQuality ? (

<div className="px-2 py-1 rounded

bg-red-500/10 border border-red-400

text-red-600 font-mono text-sm">

---

</div>

) : (

<div className="font-mono text-sm

text-gray-900">

{(lv.value as number)

.toFixed(decimals)}

{tag.unit && (

<span className="text-gray-500

text-xs ml-1">

{tag.unit}

</span>

)}

</div>

)}

</div>

);

}

### Rendering Rules

lv.value === null: Renders "---" with red tint and red border. This is the bad quality state.

lv.value !== null: Renders the numeric value with .toFixed(decimals). Decimals are resolved from tag.meta "format" field (root-to-leaf, first match wins, default 2).

tag.unit: If present, shown as a suffix in muted text (e.g., "W", "kPa", "°C").

## 8.4 Meta Field Resolution

The decimal places and unit come from the tag’s meta array, resolved at the server during tag map construction:

// tag-map.ts — getMetaField()

function getMetaField<T>(

meta: MetaLevel[], key: string

): T | null {

for (const level of meta) {

const val = level.fields[key];

if (val !== undefined

&& val !== null)

return val as T;

}

return null;

}

This walks meta[0] (root) to meta[last] (leaf). The first level containing the field wins. Applied to eng_min, eng_max, unit, and format.

The client-side resolveDecimalPlaces does the same walk on the meta array delivered in the TagDef:

// shared/utils.ts — resolveDecimalPlaces()

export function resolveDecimalPlaces(

tag: TagDef

): number {

for (const level of tag.meta) {

if (typeof level.fields['format']

=== 'number') {

return level.fields['format'] as number;

}

}

return 2;  // default

}

# 9. Threading and Safety Model

The entire server runs on a single Node.js event loop thread. There are no worker threads, no shared memory, and no locks. This is a deliberate architectural decision.

## 9.1 Why No Locks?

In a multi-threaded system, the LKV cache would need a read-write lock because the MQTT callback writes to it while the WS tick reads from it. In Node.js, these never execute concurrently:

MQTT message callback: Runs synchronously on the event loop. Writes to LKV via lkv.set(). Cannot be interrupted by the WS tick.

WS tick callback: Runs synchronously on the event loop via setInterval. Reads from LKV via lkv.getGeneration() and lkv.getValue(). Cannot be interrupted by MQTT callbacks.

DB flush timer: Runs synchronously via setInterval. Reads from the queue via queue.splice(). Cannot be interrupted.

Each callback runs to completion before the next one starts. This is the Node.js concurrency guarantee: no preemption within the synchronous phase of a callback.

## 9.2 The Async Boundary

The only async operations are:

WebSocket.send(): Buffers internally. The send() call returns synchronously; actual TCP transmission is handled by the kernel.

MQTT.publish(): Same buffering behavior.

loadTagMap(): Database query at startup only. By the time the server starts handling telemetry, the tag map is fully loaded and immutable.

No async operations occur between reading the LKV and sending the DELTA. The tick() method is entirely synchronous: it iterates clients, reads generations, reads values, and calls ws.send() — all in one unbroken run.

# 10. Configuration Reference

| Env Var | Default | Used By | Effect on Telemetry Path |
| --- | --- | --- | --- |
| MQTT_URL | mqtt://localhost:1883 | mqtt-bridge.ts | Broker address. Must match the Mosquitto instance. |
| WS_TICK_MS | 125 | ws-server.ts | WebSocket delta tick interval. 125 ms = 8 Hz. Lower = more responsive, higher CPU. Higher = less traffic, more latency. |
| DB_TICK_MS | 1000 | index.ts | DB pipeline flush interval. Placeholder. Real value depends on TimescaleDB write performance. |
| WATCHDOG_TIMEOUT_MS | 5000 | mqtt-bridge.ts | Time without telemetry before writing null to LKV for all module tags. |
| HEARTBEAT_INTERVAL_MS | 3000 | mqtt-bridge.ts | Frequency of heartbeat messages published to caro/{module_id}/beat. |